UV Scanner: Flame scanner shall be U.L. recognized. Scanner housing shall be made of rugged anodized aluminum, have two status LEDs, and connect by means of a military-style quick disconnect fitting.
Physical Desciption: Scanner shall be made of high strength anodized aluminum and mount by means of industry standard 1/2” NPTF connection and include:

- 3/8” Purge connection

- 5 Pin military-style quick disconnect cable fitting

Electrical Characteristics: Scanner shall be powered by 120 VAC or 24 VDC input power. A 120/230 VAC relay contact shall prove flame. In addition, a flame intensity signal shall be provided with the following outputs depending on the model:

- 4-20 mA

- 0 to 10 VDC

- Frequency Modulation

Manufacturer: Scanner shall be model 5002-01NC manufactured by Preferred Instruments of Danbury, CT.